BearingBench

Window Size Analysis & Model Input Design Report

XJTU-SY Rolling Element Bearing Dataset

CNN vs LSTM vs BiLSTM vs Transformer — Fault Classification

# 1. Dataset Overview

The XJTU-SY dataset consists of 15 rolling element bearings run to failure under three working conditions. Each 1.28-second snapshot is captured every minute at a fixed sampling frequency of 25,600 Hz, producing 32,768 samples per CSV file.

| Condition | RPM | Load (kN) | Bearings | Total CSVs |
| --- | --- | --- | --- | --- |
| WC1 | 2100 | 12 | 4 used (B1_1 to B1_4) | 564 |
| WC2 | 2250 | 11 | 5 used (B2_1 to B2_5) | 1,566 |
| WC3 | 2400 | 10 | 5 used (B3_1 to B3_5) | 7,034 |
| Total | — | — | 14 bearings | 9,164 files |

Note: Bearing1_5 (combined Inner+Outer fault) was excluded due to ambiguous labeling. Bearing3_2 (combined fault) was retained and labeled as Outer Race (dominant fault type).

# 2. Signal Properties

| Property | Value |
| --- | --- |
| Sampling frequency | 25,600 Hz (25.6 kHz) |
| Duration per CSV | 1.28 seconds |
| Samples per CSV | 32,768 rows + 1 header = 32,769 lines |
| Channels | 2 (Horizontal + Vertical vibration) |
| Snapshot interval | Every 1 minute |
| Hardware | National Instruments USB-3250 |

# 3. Window Size Derivation

## 3.1  Chosen Parameters

| Parameter | Value | Rationale |
| --- | --- | --- |
| Window size | 1,024 points | ~1.5 shaft revolutions |
| Stride | 512 points | 50% overlap |
| Duration | 40 ms | 1024 / 25600 Hz |

## 3.2  Windows Per CSV Calculation

windows = floor((32,768 - 1,024) / 512) + 1 = 63 windows per CSV

Total windows = 9,164 files × 63 = 577,332 samples

# 4. Model-Specific Input Design

| Model | Input Shape | Transformation | Feature Extraction | Key Strength |
| --- | --- | --- | --- | --- |
| CNN | (B,32,32,2) | Reshape | Conv2D spatial filters | Full curve shape at once |
| LSTM | (B,1024,2) | None | Hidden state memory | Temporal order preserved |
| BiLSTM | (B,256,2) | None | Bidirectional hidden state | Forward + backward context |
| Transformer | (B,256,64) | Linear + Pos. Enc. | Multi-Head Self-Attention | Global timestep relationships |

# 5. Fault Label Assignment (4 Classes)

| Label | Class | Bearings | Assignment Rule |
| --- | --- | --- | --- |
| 0 | Normal | All bearings | First 80% of CSV files |
| 1 | Inner Race | Bearing2_1, Bearing3_3, Bearing3_4 | Last 20% of CSV files |
| 2 | Outer Race | B1_1-4, B2_2/4/5, B3_1/2/5 | Last 20% of CSV files |
| 3 | Cage | Bearing1_4, Bearing2_3 | Last 20% of CSV files |

# 6. Benchmark Results

## 6.1  Final Comparison Table

| Model | Accuracy | Macro F1 | Cage Recall | Cage AUC | Time |
| --- | --- | --- | --- | --- | --- |
| LSTM | 96.6% | 0.964 | 1.000 | 1.000 | 12.9 hrs |
| CNN | 96.2% | 0.959 | 1.000 | 1.000 | 1.2 hrs |
| BiLSTM | 90.9% | 0.916 | 0.986 | 0.999 | 9.7 hrs |
| Transformer | 63.8% | 0.634 | 0.918 | 0.986 | 5.2 hrs |

Key Finding: LSTM outperforms CNN on the large XJTU-SY dataset (~190K samples). Bearing vibration signals contain periodic fault impacts — a fundamentally sequential pattern that LSTM captures better than spatial 2D image filters.

# 7. Confusion Matrices

## 7.1  LSTM — Best Model (96.6% Accuracy)

Figure 1: LSTM Confusion Matrix — Zero cross-fault confusion. Cage recall = 1.00.

## 7.2  CNN (96.2% Accuracy)

Figure 2: CNN Confusion Matrix — Cage recall = 1.00. All errors are Normal misclassifications.

## 7.3  BiLSTM (90.9% Accuracy)

Figure 3: BiLSTM Confusion Matrix — Cage recall = 0.986. Trained with SEQ_LEN=256.

## 7.4  Transformer (63.8% Accuracy)

Figure 4: Transformer Confusion Matrix — Significant Normal misclassification. Needs larger dataset or pretraining.

# 8. ROC Curves — LSTM

The LSTM ROC curves demonstrate near-perfect separation for all 4 fault classes. The Cage class achieves AUC=1.000 despite being the rarest class (1.4% of files), confirming that CLASS_WEIGHT=15.0 successfully compensated for class imbalance.

Figure 5: LSTM ROC Curves — Normal (0.990), Inner Race (0.998), Outer Race (0.998), Cage (1.000).

# 9. Interactive Dashboard

The BearingBench Flask dashboard provides real-time visualization of bearing data from MySQL and live fault prediction from uploaded CSV files. All 4 trained models are available for inference.

Figure 6: BearingBench Dashboard — Live prediction showing Normal (99.0% confidence) on Bearing1_1 file 1.csv using LSTM model.

Dashboard features:

- Metric cards: Best accuracy · Total bearings · CSV files · Cage recall

- Live fault prediction: Upload any CSV → select model → confidence bars for all 4 classes

- Model benchmark table: All 4 models with color-coded performance badges

- Bearing lifetime timeline: Click any bearing to see run-to-failure fault progression

- Severity index chart: WC1/WC2/WC3 comparison (F × RPM²)

- Per-class recall: All 4 models × 4 classes side by side

BearingBench · XJTU-SY Dataset · CNN vs LSTM vs BiLSTM vs Transformer · RTX 4060 GPU