BearingBench

Benchmark Results — Section 8 Addendum

CNN vs LSTM vs BiLSTM vs Transformer · XJTU-SY Dataset

# 8. Benchmark Results

All four models were trained on the XJTU-SY dataset with the sliding window parameters defined in Section 3. Results are reported on the held-out test set (15% stratified split).

## 8.1  Final Benchmark Table

| Model | Accuracy | Macro F1 | Cage Recall | Cage AUC | Time |
| --- | --- | --- | --- | --- | --- |
| LSTM | 96.6% | 0.964 | 1.000 | 1.000 | 12.9 hrs |
| CNN | 96.2% | 0.959 | 1.000 | 1.000 | 73 min |
| BiLSTM | 90.9% | 0.916 | 0.986 | 0.999 | 9.7 hrs |
| Transformer | 63.8% | 0.634 | 0.918 | 0.986 | 5.2 hrs |

## 8.2  Key Findings

Finding 1 — LSTM outperforms CNN on large bearing datasets:

- With 190K samples LSTM learns temporal fault impact periodicity better than CNN spatial filters

- Vibration signals contain periodic impacts at ball pass frequency — a fundamentally sequential pattern

- CNN accuracy/time tradeoff (96.2% in 73 min) makes it the practical choice for deployment

Finding 2 — Transformer underperforms due to dataset size:

- Self-attention requires significantly more data than LSTM to generalize (BERT: 3.3 billion tokens)

- With SEQ_LEN=256, attention patterns are too short to capture fault periodicity

- CWRU pretraining would likely close the gap — recommended for future work

Finding 3 — Cage fault detection is perfect for top 2 models:

- Cage recall = 1.00 for both LSTM and CNN despite being the rarest class (1.4% of files)

- CLASS_WEIGHT=15.0 successfully compensated for class imbalance

- Zero cross-fault confusion — Inner Race never predicted as Outer Race or Cage

## 8.3  Comparison with BatteryBench

| Dataset | Samples | CNN | LSTM | Transformer |
| --- | --- | --- | --- | --- |
| BatteryBench | 636 | 96% ✓ | 57% | 75% |
| BearingBench | ~190K | 96% | 97% ✓ | 64% |

Conclusion: Small datasets favor CNN. Large datasets favor LSTM. Transformer requires even more data.

BearingBench — Window Size & Results Report  |  XJTU-SY Dataset  |  CNN vs LSTM vs BiLSTM vs Transformer