BearingBench

Window Size Analysis & Model Input Design Report

XJTU-SY Rolling Element Bearing Dataset

CNN vs LSTM vs Transformer — Fault Classification

# 1. Dataset Overview

The XJTU-SY dataset consists of 15 rolling element bearings run to failure under three working conditions (WC1, WC2, WC3). Each 1.28-second snapshot is captured every minute at a fixed sampling frequency of 25,600 Hz, producing 32,768 samples per CSV file.

| Condition | RPM | Load (kN) | Bearings | Total CSVs |
| --- | --- | --- | --- | --- |
| WC1 | 2100 | 12 | 5 (B1_1 to B1_5) | 616 |
| WC2 | 2250 | 11 | 5 (B2_1 to B2_5) | 1566 |
| WC3 | 2400 | 10 | 5 (B3_1 to B3_5) | 7034 |
| Total | — | — | 15 bearings | 9,216 files |

# 2. Signal Properties

Each CSV file contains two vibration channels recorded simultaneously:

- Horizontal_vibration_signals — Channel 0

- Vertical_vibration_signals   — Channel 1

| Property | Value |
| --- | --- |
| Sampling frequency | 25,600 Hz (25.6 kHz) |
| Duration per CSV | 1.28 seconds |
| Samples per CSV | 32,768 rows + 1 header = 32,769 lines |
| Channels | 2 (Horizontal + Vertical) |
| Snapshot interval | Every 1 minute |
| Hardware | National Instruments USB-3250 |

# 3. Window Size Derivation

## 3.1  Why Windowing?

Each CSV file (32,768 samples) is too long to feed directly into LSTM or Transformer models. Sliding windows create multiple shorter fixed-length samples from each file, increasing the training dataset size while preserving temporal structure.

## 3.2  Chosen Parameters

| Parameter | Value | Rationale |
| --- | --- | --- |
| Window size | 1,024 points | ~1.5 shaft revolutions |
| Stride | 512 points | 50% overlap |
| Overlap | 512 points (50%) | Data augmentation |
| Duration | 40 ms | 1024 / 25600 Hz |

## 3.3  Physical Justification

At each working condition, one shaft revolution takes:

WC1 (2100 RPM): T = 60/2100 = 28.6 ms per revolution

WC2 (2250 RPM): T = 60/2250 = 26.7 ms per revolution

WC3 (2400 RPM): T = 60/2400 = 25.0 ms per revolution

A window of 1,024 samples = 40 ms captures:

WC1: 40 / 28.6 = 1.40 revolutions

WC2: 40 / 26.7 = 1.50 revolutions

WC3: 40 / 25.0 = 1.60 revolutions

This ensures at least one complete fault impact cycle (ball pass frequency) is captured in every window across all three working conditions.

## 3.4  Windows Per CSV Calculation

windows = floor((total_samples - window_size) / stride) + 1

= floor((32,768 - 1,024) / 512) + 1

= floor(31,744 / 512) + 1

= 62 + 1

= 63 windows per CSV

## 3.5  Total Dataset Size

Total windows = CSV files x windows per CSV

= 9,216 x 63

= 580,608 samples

# 4. Model-Specific Input Design

The same sliding window produces the raw data. Each model then transforms it differently:

## 4.1  CNN — 2D Image Input

| Property | Detail |
| --- | --- |
| Raw window | (1024, 2) |
| Transformation | reshape: (1024, 2) -> (32, 32, 2) |
| CNN input shape | (batch, 32, 32, 2) |
| Channel 0 | Horizontal vibration folded into 32x32 grid |
| Channel 1 | Vertical vibration folded into 32x32 grid |
| Why it works | Conv2D detects 2D spatial patterns in the folded signal shape |

Row 1  : samples 0-31

Row 2  : samples 32-63

...

Row 32 : samples 992-1023

## 4.2  LSTM / BiLSTM — Sequential Input

| Property | Detail |
| --- | --- |
| Raw window | (1024, 2) |
| Transformation | None — used directly as sequence |
| LSTM input shape | (batch, 1024, 2) |
| timesteps | 1024 — processed left to right |
| features | 2 (Horizontal + Vertical) |
| BiLSTM addition | Also processes right to left — captures end-of-window fault signatures |
| Limitation | May forget early timesteps for long sequences of 1024 steps |

## 4.3  Transformer — Attention-based Input

| Property | Detail |
| --- | --- |
| Raw window | (1024, 2) |
| Transformation | Linear projection: (1024, 2) -> (1024, d_model=64) |
| + Positional Encoding | Sine/cosine encoding injected at each timestep |
| Transformer input | (batch, 1024, 64) |
| Attention | Every timestep attends to ALL 1024 timesteps simultaneously |
| Key advantage | No forgetting — timestep 1 can directly influence timestep 1024 |
| After blocks | GlobalAveragePooling: (1024, 64) -> (64,) |

# 5. Model Input Comparison

| Model | Input Shape | Transformation | Feature Extraction | Key Strength |
| --- | --- | --- | --- | --- |
| CNN | (B,32,32,2) | Reshape | Conv2D spatial filters | Full curve shape visible at once |
| LSTM | (B,1024,2) | None | Hidden state memory | Temporal order preserved |
| BiLSTM | (B,1024,2) | None | Bidirectional hidden state | Forward + backward context |
| Transformer | (B,1024,64) | Linear + Pos. Enc. | Multi-Head Self-Attention | Global timestep relationships |

# 6. Fault Label Assignment (4 Classes)

Labels are assigned based on bearing identity and time position within the run-to-failure lifetime:

| Label | Class | Bearings | Assignment Rule |
| --- | --- | --- | --- |
| 0 | Normal | All bearings | First 80% of CSV files per bearing |
| 1 | Inner Race | Bearing2_1, Bearing2_4 | Last 20% of CSV files |
| 2 | Outer Race | B1_1-5, B2_2, B2_3, B3_1-5 | Last 20% of CSV files |
| 3 | Cage | Bearing2_5 | Last 20% of CSV files |

# 7. Summary

| Parameter | Value |
| --- | --- |
| Sampling frequency | 25,600 Hz (fixed across all WC) |
| Samples per CSV | 32,768 |
| Window size | 1,024 points = 40 ms |
| Stride | 512 points (50% overlap) |
| Windows per CSV | 63 |
| Total CSV files | 9,216 |
| Total windows | 580,608 samples |
| Classes | 4 (Normal / Inner Race / Outer Race / Cage) |
| CNN input shape | (batch, 32, 32, 2) |
| LSTM/BiLSTM shape | (batch, 1024, 2) |
| Transformer shape | (batch, 1024, 64) after projection |

BearingBench — Window Size Report  |  XJTU-SY Dataset  |  CNN vs LSTM vs Transformer